[page 1]

Title Page

Title: Canal Cordon Report 2025 Report on trends in mode share of vehicles and people crossing the Canal 2006-2025
[page 2]

Table of Contents

1. Introduction	3

1.1 Background to data collection	3

1.2 Definition of the canal cordon	3

1.3 Data Sources	4

1.3.1 At Work – CSO – Central Statistics Office	5

1.3.2 CSO Labour Force Survey – Employment Series Q4 2025	6

2. Traffic Surveys – Vehicles, Cyclist, Pedestrian	7

2.1 Overview	7

2.2 Numbers of vehicles, cyclists and pedestrians crossing the canal cordon by mode	9

2.2.1 Buses – Number of Buses Crossing Canal Cordon in AM Peak Period 2006 – 2025	9

2.2.2 Cars – Number of Cars Crossing Canal Cordon in AM Peak Period 2006 – 2025	9

2.2.3 Taxis – Number of Taxis Crossing Canal AM Peak Period 2006-2025	9

2.2.4 Pedestrians – Number of Pedestrians Crossing Canal Am Peak Period 2006-2025	10

2.2.5 Cyclists – Number of Cyclists Crossing Canal Cordon in AM Peak Period 2006-2025	10

2.2.6 Goods Vehicles – Number of Goods Vehicles Crossing Canal	10

2.2.7 Motor Bikes – Number of Motor Bikes Crossing Canal	11

3. Traffic and Transport Surveys – Overall Movements	11

3.1 Overview	11

3.2 Percentage mode share of people crossing the canal cordon	14

3.3 Trips Crossing the Canal Cordon by Active Travel & Public Transport Modes	16

4. Commentary on Canal Cordon Trends	17

4.1 Overall Trends	17

4.2 Public Transport Usage	18

4.3 Mode Trends	18

[page 3]

## 1. Introduction

### 1.1 Background to data collection

Since 1980, Dublin City Council (DCC) has been conducting traffic counts at 33 locations around the cordon formed by the Royal and Grand Canals. The counts are conducted during the month of November each year. Since 1997, the counts have been conducted over the AM peak period between 07:00 and 10:00.

Between 1997 and 2009, the Dublin Transportation Office (DTO) collected data from a number of sources on people crossing the Canal Cordon into Dublin’s City Centre in the AM peak period between 07:00 and 10:00. The National Transport Authority (NTA) subsumed the DTO in 2009, and has continued to collate this data on an annual basis.

Combining the sets of data enables the tracking of trends in the modes of travel that people are using to travel into the City Centre for the period 2006-2025.

### 1.2 Definition of the canal cordon

The Canal Cordon has 33 locations on the Cordon where data is collected annually on the movement of people in the AM peak

[page 4]

period between 7:00 and 10:00. As the name suggests, the cordon has been chosen to ensure (as far as possible) that any person entering the City Centre from outside must pass through one of the 33 locations where the surveys were undertaken. It should be noted that the data as presented in this report refers to movements of people in one direction only (i.e. inbound into the city centre) across the various cordon points.

There are 33 cordon points in total. All of them are located on routes used by general traffic entering the city centre.

Out of these, 22 cordon points are on bus routes into the city, and 11 are on routes that do not have buses all located around the city centre.

People using DART and suburban rail services enter the city centre through several cordon points:

- The first is at Grand Canal Street Upper. This is on the east side of the city, near Ringsend, just south of the River Liffey.

- The second is at St. John’s Road West. This is on the west side of the city centre, south of the River Liffey, near Heuston Station.

- The third is at Ossary Road. This is north-east of the city centre, in the North Wall area.

People travelling on the two Luas lines cross the cordon at two locations:

- The first is at Charlemont Street, south of the city centre.

- The second is at South Circular Road, to the west of the city centre, in Kilmainham.

There is a map showing these various 33 cordon points located around Dublin city centre.

[page 5]

### 1.3 Data Sources

Data on the movement of people across the Canal Cordon has been assembled from a number of sources which is outlined in the bullet points below:

- Dublin City Council has undertaken surveys at the Canal Cordon in November annually since 1980. Surveys are undertaken over two days at each location and an average across the two days is reported. The survey counts pedestrians, cyclists, cars, taxis, buses, goods vehicles and motorbikes crossing the cordon points in the inbound direction in the three hour, AM peak period 07:00-10:00.

- To complement the Dublin City Council Canal Cordon annual surveys, Dublin Bus have undertaken their own surveys annually on a single day at each location in November. This is not necessarily the same day as the DCC cordon counts. Since 1997 this survey has counted the number of passengers on all buses (including privately operated bus services) crossing inbound over the canal cordon points. For passenger counts, surveyors board all Dublin Bus services at the cordon point and conduct a count of passengers. For non-Dublin Bus services (such as Bus Éireann

and privately operated services) experienced surveyors estimate the volume of passengers on board as the bus crosses the cordon point. This survey is undertaken at the 22 cordon points that are on bus routes into the city.

- Since 2012, Iarnród Éireann (Irish Rail) has undertaken a census of passengers boarding and alighting on all services passing through all stations in the national rail network on a single day. In 2024, the national rail census was carried out in November. Prior to 2012 and since 1997, Iarnród Éireann had undertaken a similar passenger

[page 6]

census for services operating within the Greater Dublin Area (GDA). When the Census was GDA only, passengers who began their trip outside of the GDA would still be counted once they completed their trip within the GDA. For example, a passenger travelling from Cork to Dublin would be counted crossing the Cordon. Analysis of this data enables a calculation of the numbers of rail passengers crossing the three Canal Cordon points (inbound) between 07:00 and 10:00 on the census day.

- Transport Infrastructure Ireland (TII) (previously the Rail Procurement Agency (RPA)) undertakes an annual census of passengers boarding and alighting at all Luas tram stops. This census is undertaken in November. It has been undertaken every year since both Luas lines became operational in 2004. This data enables calculation of the number of Luas passengers crossing the two Canal Cordon points (inbound) between 07:00 and 10:00 on census day.

#### 1.3.1 At Work – CSO – Central Statistics Office

A new question was included in Census 2022 asking people whether they ever worked from home and if so, for how many days per week. Nearly 750,000 people reported that they worked from home for at least one day a week.

- More than half of workers (57%) said that they never worked from home.

- Among people who worked from home, 15% did so for one day a week, 17% for two days, 16% for three days, 10% for four days and 33% for five or more days per week.

[page 7]

Dublin City had the largest increase in the number of people working mainly from home, up by nearly 30,000 workers since 2016 and has the second highest

proportion of workers who ever worked from home, with 40% working one or more days a week from home.

#### 1.3.2 CSO Labour Force Survey – Employment Series Q4 2025

For context, respondents to the CSO’s quarterly Labour Force Survey were asked about the extent to which they have done any work at home for their job (Background Notes Labour Force Survey Quarter 4 2025 Central Statistics Office)

Almost two-thirds of those in employment in Q4 2025 stated that they never work at home (1,865,600).

Of the 956,700 people who reported that they work from home at least some of the time in Q4 2025, 530,500 said that they usually (more than half the time) work from home, down from a peak of 846,700 in Q1 2021.

There were 426,300 people who sometimes (at least one hour in the reference period) worked from home in Q4 2025, down from 432,800 in Q4 2024.

An estimated 2,833,100 persons were in employment in Q4 2025, up 2.0% (56,700) from 2,776,400 in Q4 2024.

Key findings:

- The employment rate for people aged 15-64 years was 74.5% in Q4 2025, up from 74.3% in Q4 2024.

- The number of people aged 15-89 years in employment rose by 56,700 or 2.0% to 2,833,100 people in the 12 months to Q4 2025.

- The estimated Labour Force stood at 2,961,300 in Q4 2025, up 2.4% (+68,800) from Q4 2024.

[page 8]

- The estimated total number of hours worked per week in Q4 2025 was 88.1 million, an increase of 2.5 million hours or 2.9% when compared with Q4 2024.

Source: Key Findings Labour Force Survey Quarter 4 2025 Central Statistics Office

In light of these changes the need to travel for work has changed and the effect of this change has most likely influenced some of the fluctuations observed between 2019 and 2025 in the cordon.

## 2. Traffic Surveys – Vehicles, Cyclist, Pedestrian

### 2.1 Overview

This chapter of the report records the data collected from the traffic counts only, which records the numbers of vehicles of different types and the numbers of cyclists and pedestrians. It does not include the public transport surveys which supplements the traffic counts with the additional passenger numbers on the various modes of public transport. That information is included in Chapter 3 of this report.

Table 1 below presents the total numbers of vehicles, pedestrians and cyclists crossing the Canal Cordon inbound between 07:00am and 10:00am from 2006 to 2025.

Table 1.

| Year | Bus | Car | Taxi | Walk | Cycle | Goods Vehicles | Motor Bike |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2006 | 1,680 | 58,664 | 3,825 | 17,114 | 4,839 | 2,291 | 2,395 |
| 2007 | 1,740 | 58,686 | 4,583 | 18,594 | 5,676 | 1,445 | 2,429 |
| 2008 | 1,814 | 58,897 | 5,079 | 18,360 | 6,143 | 1,223 | 2,375 |
| 2009 | 1,704 | 58,232 | 4,980 | 14,618 | 6,326 | 1,087 | 2,060 |
| 2010 | 1,688 | 58,047 | 4,809 | 15,092 | 5,952 | 993 | 1,656 |
| 2011 | 1,539 | 55,745 | 4,862 | 14,551 | 6,870 | 1,176 | 1,485 |
| 2012 | 1,503 | 55,343 | 5,277 | 17,070 | 7,943 | 1,099 | 1,425 |
| 2013 | 1,539 | 54,458 | 5,458 | 17,495 | 9,061 | 1,045 | 1,423 |
| 2014 | 1,504 | 53,033 | 4,955 | 19,711 | 10,349 | 1,087 | 1,372 |
| 2015 | 1,528 | 53,064 | 4,699 | 18,727 | 10,893 | 1,096 | 1,390 |
| 2016 | 1,652 | 51,908 | 4,779 | 21,473 | 12,089 | 1,093 | 1,464 |
| 2017 | 1,637 | 50,158 | 4,098 | 24,936 | 12,447 | 1,024 | 1,532 |
| 2018 | 1,837 | 48,820 | 4,399 | 23,858 | 12,227 | 1,153 | 1,477 |
| 2019 | 1,852 | 46,388 | 4,292 | 24,691 | 13,131 | 983 | 1,485 |
| 2020 | 1,683 | 35,041 | 2,264 | 9,235 | 4,756 | 1,045 | 581 |
| 2021 | 1,663 | 39,088 | 3,055 | 13,103 | 7,597 | 980 | 782 |
| 2022 | 1,642 | 40,207 | 3,237 | 16,951 | 9,486 | 933 | 928 |
| 2023 | 1,669 | 39,698 | 3,324 | 17,820 | 9,406 | 893 | 848 |
| 2024 | 1,721 | 38,930 | 3,919 | 18,564 | 10,646 | 893 | 797 |
| 2025 | 1,772 | 38,696 | 4,242 | 17,531 | 9,731 | 815 | 734 |

[page 9]

This section also shows a line chart depicting the total numbers of vehicles, pedestrians and cyclists crossing the Canal Cordon inbound between 07:00-10:00 from 2006 to 2025.

[page 10]

The line chart shows how people travel into the city between 2006 and 2025, using different modes of transport including car, walking, cycling, bus, taxi, goods vehicles and motorbikes.

Since 2006, car travel has seen a steady decrease in mode share of travel, with a sharp drop in 2020, followed by a partial recovery in the following years, but levels continue to decline from 2020 onwards with a shift towards active travel and public transport modes.

Walking and cycling increase over time, showing steady growth in active travel. There is a clear drop across all modes of travel in 2020, followed by a gradual recovery.

Walking and cycling continue to rise after 2020 until 2024, before levelling off and then falling slightly in 2025.

Bus and taxi use remain much lower than car travel and are also lower than walking and cycling. However, both bus and taxi numbers show a steady increase from 2020 onwards.

Motorcycles and goods vehicles are the least used modes throughout the period, with very little change compared to other types of travel.

The next sections provide an analysis of this data by mode of travel, identifying the trends in the number of vehicles, pedestrians and cyclists crossing the canal cordon during the AM peak period from 07:00-10:00. In Chapter 3, this analysis

is supplemented with additional public transport patronage data to provide a full picture of the travel trends in person terms across the canal cordon.

[page 11]

### 2.2 Numbers of vehicles, cyclists and pedestrians crossing the canal cordon by mode

#### 2.2.1 Buses – Number of Buses Crossing Canal Cordon in AM Peak Period 2006 – 2025

This section shows a bar chart of the total number of buses and both Dublin Bus and other bus separately crossing the Canal Cordon for the years 2006 to 2025.

Between 2019 and 2025, there was an overall decrease in the number of buses crossing the cordon from 1,852 to 1,772. However, within this total, Dublin Bus vehicle numbers increased by 3% whereas buses operated by Bus Éireann and private operators have displayed a decreasing trend of 18%.

In the period 2006 - 2025, the total number of buses crossing the cordon has increased by 5%.

#### 2.2.2 Cars – Number of Cars Crossing Canal Cordon in AM Peak Period 2006 – 2025

This section shows a bar chart of the total number of cars crossing the Canal Cordon for the years 2006 to 2025.

There was a decrease in the number of cars crossing the cordon from 46,388 to 38,696 between 2019 and 2025. This represents a decrease of 17%.

In the period 2006-2025 the peak year for cars crossing the canal cordon was in 2008 with 58,897 vehicles. The 2025 figure represents a decrease of 34%, or 20,201 cars, since this peak.

#### 2.2.3 Taxis – Number of Taxis Crossing Canal AM Peak Period 2006-2025

This section shows a bar chart of the total number of taxis crossing the Canal Cordon for the years 2006 to 2025.

[page 12]

Taxis made up 6.12% of all vehicles crossing the canal cordon in 2006. This proportion increased to 8.47% in 2019. Although the proportional percentage increased between 2006 and 2019, 2025 saw a drop in the number of taxis crossing the cordon from 2019, down by 1% or 50 vehicles.

#### 2.2.4 Pedestrians – Number of Pedestrians Crossing Canal Am Peak Period 2006-2025

This section shows a bar chart of the total number of pedestrians crossing the Canal Cordon for the years 2006 to 2025.

The number of pedestrians crossing the canal cordon has decreased from 24,691 in 2019 to 17,531 in 2025, a decrease of 29% or 7,160 people. In the period 2006 to 2025, there has been a 2% increase in the number of pedestrians crossing the cordon during the AM peak period.

#### 2.2.5 Cyclists – Number of Cyclists Crossing Canal Cordon in AM Peak Period 2006-2025

This section shows a bar chart of the total number of cyclists crossing the Canal Cordon for the years 2006 to 2025.

There has been a decrease in cyclists crossing the canal between 2019 and 2025 with numbers decreasing by 26% in the AM peak period. There had been a steady year on year growth in the number of cyclists crossing the cordon between 2010-2019. In 2025, despite a steady recovery in numbers following the COVID restrictions, the overall number of cyclists observed crossing the canal was 9,731 in the AM peak period, which is circa 3,400 below the 2019 peak. However, even with these lower numbers in 2025, this still represents a significant growth of 101% when compared with 2006.

[page 13]

#### 2.2.6 Goods Vehicles – Number of Goods Vehicles Crossing Canal

This section shows a bar chart of the total number of goods vehicles crossing the Canal Cordon for the years 2006 to 2025.

With the exception of 2018, the number of goods vehicles crossing the Canal Cordon in the AM Peak had remained relatively static over recent years. In 2025, there was a slight decrease of 17% between 2019 and 2025. The goods vehicle count in 2019 are similar to figures of just below 1,000 last seen in

2010. Overall, the volume of goods vehicles crossing the cordon has remained broadly unchanged since 2009. Over the longer period from 2006 to 2025 however, the number of goods vehicles crossing the cordon has decreased by 64% (1,476 vehicles). The majority of that decrease occurred in the period

2006-2007 and coincided with the opening of the Dublin Port Tunnel in 2006 and the implementation of the HGV Management Strategy in 2007.

#### 2.2.7 Motor Bikes – Number of Motor Bikes Crossing Canal

This section shows a bar chart of the total number of motor bikes crossing the Canal Cordon for the years 2006 to 2025.

There has been a decrease of 51% in the number of motor bikes crossing the canal cordon between 2019 and 2025. In the period 2006 - 2025, the volume of motor cyclists crossing the cordon in the AM peak has fallen by roughly 69%, which equates to 1,661 vehicles. The declining trend seemed to have stabilised since 2011 until the occurrence of COVID-19.

[page 14]

## 3. Traffic and Transport Surveys – Overall Movements

### 3.1 Overview

While section 2 reports the number of vehicles, cyclists and pedestrians crossing the canal cordon, this chapter supplements that information with the data obtained from the public transport surveys, to give the overall number of people travelling across the cordon.

Using that supplementary data, Table 2 gives the total numbers of people crossing the canal cordon inbound in the AM peak period between 07:00-10:00 for 2025 and for each year since 2006, broken down by mode of travel.

Table 2.

| Year | Bus | Rail | Luas | All Public Transport | Car | Taxi | Walk | Cycle | Goods Vehicles | Motor Bike | Total Person Trips |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2006 | 59,874 | 33,534 | 9,029 | 102,437 | 76,850 | 1,453 | 17,114 | 4,839 | 2,291 | 2,395 | 207,379 |
| 2007 | 57,201 | 35,692 | 9,171 | 102,064 | 71,597 | 2,154 | 18,594 | 5,676 | 1,445 | 2,429 | 203,959 |
| 2008 | 60,438 | 32,324 | 9,242 | 102,004 | 67,732 | 1,930 | 18,360 | 6,143 | 1,223 | 2,375 | 199,767 |
| 2009 | 56,168 | 25,723 | 8,776 | 90,667 | 71,043 | 2,739 | 14,618 | 6,326 | 1,087 | 2,060 | 188,540 |
| 2010 | 50,420 | 23,580 | 9,111 | 83,111 | 71,978 | 2,260 | 15,092 | 5,952 | 993 | 1,656 | 181,042 |
| 2011 | 54,251 | 22,932 | 9,949 | 87,132 | 69,681 | 2,674 | 14,551 | 6,870 | 1,176 | 1,485 | 183,569 |
| 2012 | 52,007 | 23,999 | 10,014 | 86,047 | 68,626 | 3,271 | 17,070 | 7,943 | 1,099 | 1,425 | 185,454 |
| 2013 | 56,177 | 24,969 | 10,835 | 91,981 | 68,072 | 3,111 | 17,495 | 9,061 | 1,045 | 1,423 | 192,188 |
| 2014 | 56,671 | 24,866 | 11,670 | 93,207 | 64,169 | 2,775 | 19,711 | 10,349 | 1,087 | 1,372 | 192,670 |
| 2015 | 57,584 | 29,521 | 12,503 | 99,608 | 65,269 | 2,960 | 18,727 | 10,893 | 1,096 | 1,390 | 199,943 |
| 2016 | 56,572 | 31,309 | 12,254 | 100,135 | 64,885 | 2,724 | 21,473 | 12,089 | 1,093 | 1,464 | 203,863 |
| 2017 | 60,798 | 34,409 | 11,953 | 107,160 | 61,694 | 2,623 | 24,936 | 12,447 | 1,024 | 1,532 | 211,416 |
| 2018 | 64,206 | 34,471 | 13,835 | 112,512 | 60,537 | 2,156 | 23,858 | 12,227 | 1,153 | 1,477 | 213,920 |
| 2019 | 65,048 | 37,407 | 13,832 | 116,287 | 57,985 | 2,661 | 24,691 | 13,131 | 983 | 1,485 | 217,223 |
| 2020 | No Data | No Data | No Data | No Data | 43,100 | 928 | 9,235 | 4,756 | 1,045 | 581 | 59,645 |
| 2021 | 38,885 | 13,330 | 5,740 | 57,955 | 47,687 | 1,741 | 13,103 | 7,597 | 980 | 782 | 129,845 |
| 2022 | 61,362 | 25,314 | 10,982 | 97,658 | 49,053 | 2,234 | 16,951 | 9,486 | 933 | 928 | 177,243 |
| 2023 | 66,949 | 29,978 | 13,848 | 110,775 | 48,035 | 2,127 | 17,820 | 9,406 | 893 | 848 | 189,904 |
| 2024 | 65,758 | 32,241 | 13,275 | 111,274 | 48,273 | 2,116 | 18,564 | 10,646 | 893 | 797 | 192,563 |
| 2025 | 67,942 | 34,867 | 15,358 | 118,167 | 48,757 | 3,097 | 17,531 | 9,731 | 815 | 734 | 198,832 |

[Page 17]

The data in the table is also shown in a line chart displaying overall trends over time. From 2006 to 2019, the total number of person trips generally increased.

It also shows that total trips were about 207,000 in 2006, fell to a low of 181,000 in 2010, and then steadily recovered.

By 2019, trips reached a peak of over 217,000, the highest in the series. In 2020, total trips drop to around 59,600, with

[Page 18]

missing data for all public transport modes due to the COVID 19 pandemic.

Trips steadily recover from 2021 onwards, with total trips reaching 198,800 in 2025.

Public transport trends follow a similar pattern being generally stable at around 100,000 to 116,000 trips per year between 2006 and 2019, followed by a decrease between 2009 and 2012, with an increase, thereafter, reaching a peak of approximately 116,000 in 2019.

There was no public transport data recorded in 2020. Trips made by public transport then steadily recover from 2021, reaching an all time high of 118,167 in 2025.

Car travel shows an overall downward trend from 2006 to 2025, starting at 76,850 trips in 2006 to 48,757 in 2025.

On the other hand, there is a long-term increased trend towards walking and cycling.

Taxi trips fluctuate but remain low, generally between 2,000 and 3,000, except for a drop in 2020.

Goods vehicle trips gradually decline over time from about 2,300 in 2006 to under 1,000 after 2019.

Motorbike trips also decrease steadily from about 2,400 in 2006 to around 700 by 2025.

### 3.2 Percentage mode share of people crossing the canal cordon

Table 3 gives the percentage mode share for all modes of travel used by people crossing the canal cordon inbound between 07:00 and 10:00 for the years 2006 to 2025.

[page 19]

Table 3.

| Year | Bus (%) | Rail (%) | Luas (%) | All Public Transport (%) | Car (%) | Taxi (%) | Walk (%) | Cycle (%) | Goods (%) | Motor Bike (%) | Total Person Trips |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2006 | 28.9 | 16.2 | 4.4 | 49.4 | 37.1 | 0.7 | 8.3 | 2.3 | 1.1 | 1.2 | 207,379 |
| 2007 | 28.1 | 17.5 | 4.5 | 50.0 | 35.1 | 1.1 | 9.1 | 2.8 | 0.7 | 1.2 | 203,959 |
| 2008 | 30.3 | 16.2 | 4.6 | 51.1 | 33.9 | 1.0 | 9.2 | 3.1 | 0.6 | 1.2 | 199,767 |
| 2009 | 29.8 | 13.6 | 4.7 | 48.1 | 37.7 | 1.5 | 7.8 | 3.4 | 0.6 | 1.1 | 188,540 |
| 2010 | 27.9 | 13.0 | 5.0 | 45.9 | 39.8 | 1.3 | 8.3 | 3.3 | 0.5 | 0.9 | 181,042 |
| 2011 | 29.5 | 12.5 | 5.4 | 47.5 | 38.0 | 1.5 | 7.9 | 3.7 | 0.6 | 0.8 | 183,569 |
| 2012 | 28.0 | 12.9 | 5.4 | 46.4 | 37.0 | 1.8 | 9.2 | 4.3 | 0.6 | 0.8 | 185,454 |
| 2013 | 29.2 | 13.0 | 5.6 | 47.9 | 35.4 | 1.6 | 9.1 | 4.7 | 0.5 | 0.7 | 192,188 |
| 2014 | 29.4 | 12.9 | 6.1 | 48.4 | 33.3 | 1.4 | 10.2 | 5.4 | 0.6 | 0.7 | 192,670 |
| 2015 | 28.8 | 14.8 | 6.3 | 49.8 | 32.6 | 1.5 | 9.4 | 5.4 | 0.5 | 0.7 | 199,943 |
| 2016 | 27.8 | 15.4 | 6.1 | 49.1 | 31.8 | 1.3 | 10.5 | 5.9 | 0.5 | 0.7 | 203,863 |
| 2017 | 28.8 | 16.3 | 6.0 | 50.7 | 29.2 | 1.2 | 11.8 | 5.9 | 0.5 | 0.7 | 211,416 |
| 2018 | 30.0 | 16.1 | 6.5 | 52.6 | 28.3 | 1.0 | 11.2 | 5.7 | 0.5 | 0.7 | 213,920 |
| 2019 | 29.9 | 17.2 | 6.4 | 53.5 | 26.7 | 1.2 | 11.4 | 6.0 | 0.5 | 0.7 | 217,223 |
| 2020 | No data | No data | No data | No data | No data | No data | No data | No data | No data | No data | 59,645 |
| 2021 | 29.9 | 10.3 | 4.4 | 44.6 | 36.7 | 1.3 | 10.1 | 5.9 | 0.8 | 0.6 | 129,845 |
| 2022 | 34.6 | 14.3 | 6.2 | 55.1 | 27.7 | 1.3 | 9.6 | 5.4 | 0.5 | 0.5 | 177,243 |
| 2023 | 35.3 | 15.8 | 7.3 | 58.3 | 25.3 | 1.1 | 9.4 | 5.0 | 0.5 | 0.4 | 189,904 |
| 2024 | 34.1 | 16.7 | 6.9 | 57.8 | 25.1 | 1.1 | 9.6 | 5.5 | 0.5 | 0.4 | 192,563 |
| 2025 | 34.2 | 17.5 | 7.7 | 59.4 | 24.5 | 1.6 | 8.8 | 4.9 | 0.4 | 0.4 | 198,832 |

[Page 20]

The data from the table above, also depicted in a line chart shows that overall, there is a clear shift in how people travel overtime, with changes in both the total person trips and how those trips are made.

All public transport, including bus, rail and Luas combined, represents the highest mode share over all years. It starts at just under half of all trips in 2006, at 49.4%, and increases to its highest level of 59.4% by 2025.

For bus alone, it remains relatively stable, staying between 28% to 35% across all years.

Rail and Luas both show gradual increases, particularly in later years, with Luas at almost 8% in 2025 and rail at 17.5% in 2025.

Car travel shows a long-term decline. It begins at 37.1% in 2006 and falls steadily to 24.5% by 2025. This indicates a gradual shift away from car use over the period.

[page 21]

Walking and cycling both increase overall, showing growth in active travel. Walking rises from 8.3% in 2006 to between 9% to 11% in later years, with a slight decline to 8.8% in 2025.

Cycling shows stronger growth, increasing from 2.3% in 2006 to a peak of around 6.0%, before settling at 4.9% in 2025.

Taxi use remains consistently low, generally between 1% and 2%, with only small variations in mode share over time.

Goods vehicles and motorcycles are the smallest categories throughout, both remaining below 1.5% for the entire period and showing very little change.

There is a major disruption in 2020, where all percentage data is unavailable and total trips drop sharply to 59,645. After 2020, travel levels recover significantly but do not fully return to earlier peaks.

Looking at total trips, the number starts at just over 207,000 in 2006, then falls in the late 2000s. Total trips then gradually increase again to a peak of over 217,000 in 2019, followed by a sharp drop in 2020, then finally recovers to just under 199,000 by 2025.

### 3.3 Trips Crossing the Canal Cordon by Active Travel & Public Transport Modes

The table below shows the number and mode share of trips crossing the canal cordon in the AM peak period by active travel & public transport modes during

the period 2006 to 2025. Active travel & public transport modes consist of public transport, active modes (walking & cycling) and taxi.

Overall, the data shows a clear long-term shift in how people travel, with increasing reliance on active travel and public transport.

[page 22]

During the 2025 AM peak period (7am to 10am), 74.7% of all inbound trips crossing the canal cordon were made by active travel & public modes (walking, cycling, public transport or taxi). The active travel & public mode share has grown year on year since 2010 up to 2025 with the exception of 2020 and 2021 due to COVID.

In 2025, 148,526 trips crossed the cordon by active travel & public modes in the three hour AM peak period. This demonstrates a recovery in levels of mode share and person trips by active travel & public modes since COVID

This data is also presented in a doughnut chart of the mode share of people crossing the Canal Cordon by active travel and public transport and mechanised transport modes of for 2006 and 2025.

This visual compares the share of people crossing Dublin’s Canal Cordon using active travel and public transport and mechanised transport modes in 2006 and 2025, showing a clear shift towards more sustainable travel over time.

In 2006, active travel and public transport modes account for 60.7% of trips, representing 125,843 people, while unsustainable modes make up 39.3%, or 81,536 people. By 2025, the share of active travel and public transport modes has increased significantly to 74.7%, representing 148,526 people, while mechanised transport modes have decreased to 25.3%, or 50,306 people.

There are also two bar charts displaying this information for both mode share and numbers of passengers for active travel and public transport modes and mechanised transport modes.

[page 23]

## 4. Commentary on Canal Cordon Trends

### 4.1 Overall Trends

The total number of people crossing the canal cordon in the AM peak period (07:00-10:00) decreased by 8.5% between 2019 and 2025. This is a decrease of 18,391 person trips, bringing the total number of people crossing the canal

(inbound) in the AM peak period to 198,832. There had been a continual annual increase in the number of people crossing the canal in the AM peak from 2010 –

2019, this figure lowered during 2020-2021 due to COVID 19 restrictions. In 2025 there was an increase of 53.1% in overall numbers between 2021 and 2025 which equates to an additional 68,987 trips.

### 4.2 Public Transport Usage

At 118,167 the number of public transport users crossing the cordon between 07:00 and 10:00 on census day was 1.6% higher in 2025 than in 2019. Luas usage figures were higher in 2025 (15,358) compared to 2019 (13,832). Rail trips recorded in 2025 are 6.8% lower than in 2019. Bus patronage overall continues to exceed pre-Covid levels, with a total of 67,942, representing an increase of 2,894 (4.4%) relative to 2019.

### 4.3 Mode Trends

Based on the data above, a number of key changes in travel across the canal cordon may be identified.

In percentage terms, mode share for bus travel across the canal cordon in 2025 is now 34.2%. This is an increase of 4.3% on the 2019 and 2021 figures of 29.9%. In absolute

terms, bus patronage in person trips increased in 2025 relative to 2019 as it carried 67,942 people. It also increased if

[page 24]

comparing 2025 to 2021, with an additional 29,057 trips travelling into the City Centre in the AM peak period. This represents almost 58% of all public transport trips in 2025 into the City Centre in the peak period.

The mode share for rail across the canal cordon in 2025 was 17.5%. This figure is only 0.3% lower than in 2019. Intercity, Suburban Rail and DART had lost a significant share of travel into the City Centre between 2007 and 2014. However, this trend reversed in 2015 and had continued to steadily increase year on year until 2019, prior to the pandemic. Post pandemic, rail patronage has recovered year on year, with an 8.1% increase in usage in 2025 compared to 2024 (an increase in 2,626).

Car mode share (excluding taxis) decreased in 2025 to 24.5%, maintaining the trend of the year-on-year decline seen from 2010 until 2019, and again post

pandemic. Car use increased slightly following the pandemic recovery, increasing by 2.9% between 2021 and 2022, but this trend was reversed in 2023

and has continued into 2025, with a decrease of 0.6% from the 2022 figure. Compared with 2021, this represents an increase of 2.2%. It is worth noting that even with changes in pandemic related mode choices, on census day 2025 there has been a reduction of 37% in the number of cars entering the city during the AM peak period compared to census day 2006.

The total number of people walking into the city has fallen by roughly 29% between 2019 and 2025, but it is noted that the trend in recent years shows a decrease in pedestrian numbers, with a decline of 5.6% when compared to 2024. Walking levels were at their highest in 2017 since the cordon count began (11.8%) and while there was a small decline in 2018 (11.2%) it showed an upward trend up to 2019 (11.4%) until the walk

[page 25]

mode share dropped slightly to 9.6% in 2024. However, in a continuation of this trend, it has now fallen to 8.8% in 2025.

Except for a slight drop in 2018, cycling had presented a steadily increasing trend between 2006 and 2019. In 2025 it is currently represented by a mode share of 4.9%, showing a decrease from 5.5% in 2024. Overall cycle numbers are up 101% on 2006 levels.

There were 27,262 “active trips” (walking and cycling) crossing the canal cordon during the AM peak period, which is slightly lower than the total number of passengers carried by the entire heavy rail network for the same period.

Over 3,097 people entered the city by taxi in 2025, which represents a 16% increase on 2019 levels. Until 2025, the peak taxi use occurred in 2012 when over 3,271 passengers crossed the canal cordon in the AM peak period.

The number of motor bikes entering the city has decreased since 2019 (by 51%). There had been a slow and steady downward trend of motorcycle use between 2006 and 2013 with a relatively flattening from 2013 to 2019, with motorcycle mode share relatively static at 0.7% from 2013 to 2019. Since the pandemic this mode share has fallen steadily, now standing at just 0.4% in 2025.

There has been a slight decrease of 17% (168 vehicles) in the number of goods vehicles entering the city during the AM peak period between 2019 and 2025.

Goods vehicle volumes in 2025 have marginally decreased continuing a downward trend, with just over 815 vehicles crossing the canal cordon in the AM peak period, a decrease of 64% since 2006.

Since 2010, there had been a trend of increasing mode share for active travel & public transport modes, with a consistent

[page 26]

level of increase each year up to 2019. In 2025 the overall mode share for active travel & public transport modes (walking, cycling and public transport) was 74.7%, which is 3.4% higher than 2022, demonstrating recovery by maintaining a high proportion even given the unusual circumstances brought about by COVID-19. Goods vehicles and journeys by car and motorbike accounted for 25.3% of the trips crossing the canal cordon.